WriteIMEI User Guide

Version: 1.0.1

DocCode:

Date: 2014-10-24

www.spreadtrum.com

Important Notice

Copyright Notice

Copyright © 2013, Spreadtrum Communications, Inc. All rights reserved.

Trademarks

Spreadtrum Communications, Inc. and Spreadtrum Communications, Inc.'s products are exclusively owned by Spreadtrum Communications, Inc. References to other companies and their products use trademarks owned by the respective companies and are for reference purpose only.

Warranty Disclaimer

Spreadtrum Communications, Inc. makes no representations or warranties, either expressed or implied, by or with respect to anything in this document, and shall not be liable for any implied warranties of merchantability or fitness for a particular purpose or for any indirect, special or consequential damages.

Confidentiality

The information contained herein (including any attachments) is confidential. The recipient hereby acknowledges the confidentiality of this document, and except for specific purposes, this document shall not be disclosed to any third party.

About This Document

Purpose

This document describes how to write IMEI into phone.

Intended Audience

This document is intended for:

Installation and commissioning engineer

Technical support engineer

Organization

This document consists of five chapters as follows.

| Chapter | Describes |
| --- | --- |
| Overview | Introduce function, runtime environment, and so on. |
| Quick Start | Introduce basic operation of the tool. |
| Operation Guide | Introduce detail operation of the tool. |
| Configuration | Introduce some useful settings of the tool. |
| FQA | Answer and question. |

Conventions

Symbol Conventions

The symbols that may be found in this document are defined as follows.

| Symbol | Description |
| --- | --- |
|  | Indicates a hazard with a high level of risk that, if not avoided, will result in death or serious injury. |
|  | Indicates a hazard with a medium or low level of risk which, if not avoided, could result in minor or moderate injury. |
|  | Indicates a potentially hazardous situation that, if not avoided, could cause equipment damage, data loss, and performance degradation, or unexpected results. |
|  | Indicates a tip that may help you solve a problem or save time. |
|  | Provides additional information to emphasize or supplement important points of the main text. |

Frequently Used Abbreviations

| Abbreviation | Description |
| --- | --- |

GUI Conventions

| Convention | Description |
| --- | --- |
| Boldface | Buttons, menus, parameters, tabs, windows, and dialog titles are in boldface. For example, click OK. |
| > | Multi-level menus are in boldface and separated by the ">" signs. For example, choose File > Create > Folder. |

Keyboard Operation

| Format | Description |
| --- | --- |
| Key | Press the key. For example, press Enter and press Tab. |
| Key 1+Key 2 | Press the keys concurrently. For example, pressing Ctrl+Alt+A means the three keys should be pressed concurrently. |
| Key 1, Key 2 | Press the keys in turn. For example, pressing Alt, A means the two keys should be pressed in turn. |

Mouse Operation

| Action | Description |
| --- | --- |
| Click | Select and release the primary mouse button without moving the pointer. |
| Double-click | Press the primary mouse button twice continuously and quickly without moving the pointer. |
| Drag | Press and hold the primary mouse button and move the pointer to a certain position. |

Reference

Contents

1 Overview	2-1

1.1 Introduction	2-2

1.2 Runtime environment	2-2

1.2.1 Hardware	2-2

1.2.2 Software	2-2

2 Quick Start	2-1

2.1 Main User Interface	2-2

2.2 Quick Operation	2-4

2.2.1 Step by Step	2-4

3 Operation Guide	3-1

3.1 Mode Select	3-2

3.2 Set	3-2

4 Configuration	4-1

4.1 WriteIMEI.ini	4-2

5 FQA	5-4

Figures

Figure 2-1 Main user interface	2-2

Figure 2-2 Mode Select dialog	2-4

Figure 2-3 Set dialog	2-4

Figure 2-4 Test result: PASS	2-5

Figure 2-5 Test result: FAIL	2-6

Figure 2-6 Test result: time out	2-7

Figure 3-1 Set Dialog	3-4

- Overview

Introduction

WriteIMEI is a tool that can be used to read and write IMEI, SN, Bluetooth Address and WIFI Address.

WriteIMEI also can check Software Version and AP Version of the phone.

WriteIMEI supports two modes to connect with phone, “Calibration Mode” and “Normal Mode”.

Calibration Mode: Connected phone to PC with USB cable which V-BUS is disconnected, and then powered on it for test.

Normal Mode: Powered on phone normally, and then connect to PC for test.

Runtime environment

Hardware

  - A phone that needs to test.

  - A USB cable with V-BUS is disconnected (If you want to use “Calibration Mode”).

  - A normal USB cable (If you want to use “Normal Mode”).

Software

  - Microsoft Windows 2000 or XP, Vista, Win7 or advanced operation system.

Quick Start

Main User Interface

Figure 2-1 Main user interface

The major elements on the interface are described as follows:

1. PORT and Mode Select. If Mode Select is Calibration Mode, PORT must be USB. If Mode Select is Normal Mode, PORT must be COM(X).

2. Set Dialog. Some useful configuration can be set in this, please reference to 3.2 Set.

3. Operation area. Read button is for reading IMEI/SN/BT/WIFI from the phone. Write button is for writing IMEI/SN/BT/WIFI to the phone. Stop button is for stopping reading or writing operation.

4. Test result statistics. Yield is pass ratio.

5. Prompt for testing. All test information will be displayed is this area.

6. Version display area. When you checked Check Version or Check AP Version in the Set dialog, the version information will be displayed in this area after reading from phone.

7. Test items. These items can be set in the Set dialog. They are maybe generated automatically by default, or generated by inputting manually.

8. Status for connecting. All connected status will be displayed in there.

- Connect status:

- Disconnect status:

Quick Operation

Step by Step

Click “WriteIMEI.exe” to run the tool.

Select Calibration Mode in Mode Select, and then select USB in PORT.

Figure 2-2 Mode Select dialog

- If you want to use this tool to test WCDMA data card, then the Switch mode without rebooting DUT (only for WCDMA data card) must be selected.

Click Set button to open setting dialog for some basic configuration.

- If you want to know more settings, please reference to 3.2 Set.

Figure 2-3 Set dialog

Input IMEI, SN , BT or WIFI, and then press Write button to write IMEI and other items to the phone.

Test finished.

If you select Normal Mode, you must first power on the phone and connected it with PC, and then click Read or Write button to begin test.

If you select Calibration Mode, you must first click Read or Write button, and then powered on the phone.

----End

Operation Guide

Mode Select

1. Normal Mode. In this mode, the phone must be connected first, and then powered on, and to Read or Write at the last. You can check Disable Auto-USB to enable USB or not.

2. Calibration Mode. In this mode, must to click Read or Write button at fist, and then powered on the phone. The V-BUS pin of the USB cable must be disconnected. If test WCDMA data card, Switch mode tihout rebooting DUT(only for WCDMA data card) must be checked.

Set

Figure 3-1 Set Dialog

1. PhaseCheck. Phase check or not.

2. Same IMEI. When there are more than one IMEI to be written, this item will be enabled. It means IMEI1=IMEI2=IMEI3=IMEI4. For example, if IMEI1 = 36252343242096, then IMEI2=IMEI3=IMEI4= 36252343242096.

3. IMEI Generate Mode. There are two generate mode to select. Manual Input: generated by manually. Auto-Generate IMEI: generated by automatically.

4. Bluetooth and WIFI Generate Mode.

- BlueTooth Mac Address: To write Bluetooth or not.

- Generate BlueTooth Addr. Automatically: generate Bluetooth automatically or not.

- WIFI mac Address: To write WIFI or not.

- Generate WIFI Mac Addr. Automatically: generate WIFI automatically or not.

- Generate BlueTooth Mac Address. Automatically: generate by System Time or Initial Addr.

- Generate WIFI Mac Address. Automatically: generate by Auto-Generate or Initial Addr.

5. Related IMEI setting.

- Base IMEI Number: Only available when generating IMEI automatically. For example, if Base IMEI Number=36252343242096, then IMEI=36252343242096X(X is generated by crc calculate ), IMEI2=36252343242097X, IMEI3=36252343242098X, IMEI4=36252343242099X.

- IMEI CRC Check (The 15th digital): Check IMEI for CRC.

- Save IMEI: Save IMEI to txt file(IMEI.txt) or database(IMEI.mdb).

- Prevent the IMEI Reuse (Check from log file or mdb): When user input a IMEI, check if it has been exist in the txt file or database.

6. Check SW version and AP version.

- Check Version: If checked it and inputted the key words in the edit, then will check the key words with the software version read from the phone.

- Check AP Version: If checked it and inputted the key words in the edit, then will check the key words with the AP version read from the phone.

7. Related SN Setting.

- SN1: If checked it, it means to write SN1 to the phone. Only this item is checked, the Generate SN1 Automatically item can be enabling.

- Generate SN1 Automatically: If checked it, then will generate SN1 automatically, by manual input.

- SN2: If checked it, it means to write SN2 to the phone. Only this item is checked, the Generate SN2 Automatically item can be enabling.

- Generate SN2Automatically: If checked it, then will generate SN2 automatically, by manual input.

8. Check IMEI After Read: After user read IMEI from the phone, if this item is be checked, the tool will check it.

9. Operating Area.

10. Mode Selection Area

- WG: WCDMA + GSM Calibration Mode

- AP: AP Mode.

- Program Key: Enter this mode, write program key to the phone.

Configuration

WriteIMEI.ini

WriteIMEI.ini configures the common settings, such as UART port, instrument, cable loss…etc. As most of the settings could be configured by UI directly, we only describe the useful items which should be configures by manually:

[General]
Timeout					=30000

WriteTimeCheck			=0

WriteTime				=20000

WaitForAPTime			=7

CheckStationName		=CFT

CurrentStationName		=IMEI

Timeout: Specifies the wait time after DUT is under calibration mode. (Only available while calibration mode).

WriteTimeCheck: When write items to the phone, if needs to check timeout. 0: no; 1: yes.

WriteTime: The time that needs to check when write items to the phone.

WaitForApTime: Sleep WaitForApTime time and then to read AP version.

CheckStationName: The station needs to be checked.

CuurentStationName: Current station name, maybe IMEI by default.

[Control]
User Privilege				=1

Language				=1

User Privilege: 0: Operator; 1: Admin.

Language: 0: Chinese; 1: English.

[DCSOURCE]
UseDCS					=0

DCType					=2

DCGpibNo				=0

DCGpibAddr				=5

TestCounts				=1

UseDCS: Use PowerSupply.

DCType: Specifies the type of PowerSupply：0---Keithley[230X]; 1---Keithley[2304]; 2---Agilent[663XX]; 3---Agilent[E36XX].

DCGpibNo: Card number of GPIB.

DCGpibAddr: Specifies the address of GPIB.

TestCounts: Specifies the count of loop test.

FQA

Revision History

| Version | Date | Owner | Notes |
| --- | --- | --- | --- |
| 1.0.0 | 2013-02-01 | Teana.Hu | Created |
| 1.0.1 | 2014-10-24 | Teana.Hu | Modify Set dialog |