DVR-30 Upgrading Guidance

Note: Before re-burn, please charge the devices to ensure there is enough battery capacity to support re-burn. If devices out off battery during re-burn, DVR will be damaged and never turn on unless break up the dvr and re-assemble.

Please don't change the file name( only if the system guides users to change the file name ), or DVR couldn't recognize the file.

DVR-30（B733ST）：

1.Copy "卡量产工具", "ePDKv100.img" into TF card, Insert TF card into computer

2.open"卡量产工具", install “PhoenixCardSetup_V301_20111111.msi”

3.after install finish, open"卡量产工具" for choose “TF boot.img” start burn

4.after finish burn, exit the interface

5.copy “ePDKv100.img” into TFcard and insert to the device, press "power" button, when you see “正在量产” means in updating, when you see “量产完成” means success.

6.take out the card

By the way, on the process of updating, if you see "卡错误" on the screen, which means you need to do again from step 3.

1.使用读卡器插入TF卡连电脑。

2打开卡量产工具，安装名为PhoenixCardSetup_V301_20111111.msi文件。

3安装完成之后打开“卡量产工具”文件，在镜像文件行选择 TF boot. img .的文件，点击烧录。

4烧录完成之后退出界面。

5把软件“ePDKv100.img”复制到TF卡，然后把TF卡插入机器卡槽内按开关键开机升级，升级时出现“正在量产”字样附有进度条，升级完成之后会显示“量产完成”。

6关机拔卡，此时升级完成。

备注：如果在升级过程中出现“卡错误”现象，需从升级步骤中的第3步骤重新开始烧录。