Release Date: 2008/12/19

Documents Number: 0.91

MT3351

MediaTek PND Flash Tool

Revision History

Change Log

| Revision | Date | Author | Change Log |
| --- | --- | --- | --- |
| V0.54 | 2008/12/8 | Myron Li | Introduction to Flashtool usage. |
| V0.91 | 2008/12/23 | Myron Li | SOP |

Reviewer’s Comments

| Date | Reviewer |  | Comments |
| --- | --- | --- | --- |

Table of contents

Revision History	2

Table of contents	3

# MediaTek PND Flashtool

## Flashtool Introduction

MediaTek PND Flashtool is designed for MT3351 chipset flash download usage. It supports various storage types, including NOR flash, NAND flash, SD/MMC, MDOC, SPI NOR, etc.

# Flashtool Download

## Easy Mode - Direct Download

MediaTek PND Flashtool provides an easy mode, which simplifies all things that you need to tune to download images/firmware into flash storage.

Plug in the device, select suitable COM port number and baud rate speed, then simply press “Download” button. Everything will be then done!

If for some reason the download process fails, an error code will be displayed for error handling.

# Flashtool Setup

## Advance Mode

In advance mode, users can setup the profile and store it for later usage. In most cases, users are not required to do any setup things.

# Flashtool Mass Production Download

## MP Mode: 1-to-8 Downloads

MediaTek PND Flashtool provides a special mode, which lets users can download up to 8 devices at one time.

When a new device is plugged into USB port in the machine, Flashtool will detect this port automatically.

Up-to-8 devices can be plugged in sequentially in the current version. After all devices are plugged in, users can place “Download All” button to begin downloading firmware(s) to all devices simultaneously.

If any of the download processes fail, the error code will be displayed for error handling.

## Per-Device-Data (PDD)

There are some data dependent on the device, and in current design, MediaTek calls it PDD (Per-Device-Data). Under Windows CE project, those PDD data are stored in a region named “Meta-zone”.

To generate PDD, MediaTek offers a tool named “PDD Generator”, which can be customized by client. For detailed information about PDDGenerator, please refer to relative document.

Users can use PDDGenerator to generate the number they want to use for each mass production. After generating the PDD data, Flashtool can download different PDD data for each device, and notify operators the downloading results.

Note:

Please put all the PDD files under ~\Flashtool\Release\Plugin\Download_Ready\ folder, unless you have your customized version.

To use PDD ability in Flashtool Mass Production mode, you have to invoke PDDAgent under ~\Flashtool\Release\Plugin\  folder.

“PDD Agent” will do handshaking with Flashtool automatically, and notify Flashtool the information of PDD data. After Flashtool completes the download process, PDDAgent will then move those successful ones into ~\Flashtool\Release\Plugin\Download_OK\ folder, and those failed ones into ~\Flashtool_UI\Release\Plugin\Download_Fail\ folder. Operators can then recycle those failure data and try to reuse them again.

To use Flashtool MP-PDD ability, please select “43EVK WinCE MP” profile, and enter “MP Mode”. After plugging all the devices and get detected, press “Download All” button. Flashtool will then begin to do handshaking with PDDAgent. It’s suggested to run PDDAgent before beginning to download.

You can customize PDDAgent to meet your specific requests, like generating report of flash results, getting data from remote servers, etc. MediaTek provides the complete handshaking protocol for you to write your personalized PDDAgent to communicate with MediaTek PND Flashtool.

# Standard Operation and Known Issues

There are some limitations on the Flashtool usage, and please make sure you follow the standard operation steps, to make sure MediaTek PND Flashtool can run without problem.

- DO NOT remove or plug in devices during operation, like downloading, formatting, or reading-back, or multiple downloading.

(This will cause the process failure and USB detection mechanism chaos.)

- DO NOT move Flashtool application during process. Keep Flashtool application always on top.

(This will cause the flash result cannot display correctly after process finishes in MP/Multiple-Process mode.)

- BE SURE to set bootstrap pin of MT3351 device on the right way (Boot mode).

- BE SURE to re-plug the device again if you want to do Flashtool operation again after

- BE SURE to plug in devices one after one in MP/Mass-Production mode. DO NOT plug in the next device until Flashtool has successfully detected the current one.

# USB Connection

If you cannot have new plug-in device displayed on the Flashtool UI (Flashtool cannot auto-detect it), then probably the USB connection between your device(s) and PC is having trouble. Please check the following things to make sure everything is correct.

## USB Driver Installation - PNP (Plug And Play)

When a new device plugged into the machine (PC or notebook), Microsoft Windows will invoke PNP mechanism, and detect this device automatically:

PS: If for any reason you cannot have the machine detect the device, try to re-plug the device again. If you still cannot have this device detected, restart your machine.

After successfully detecting the device, Windows system will then ask you to provide the right driver for this new device.

The right device – “3351 USB VCOM Driver” will be detected and suitable USBB driver is then asked to provide.

The Flashtool comes with the device driver under ~\Driver folder.

## USB Detection

Check whether the register in your PC is in chaos status. Select “Device Manager” from “Control Panel”, and check whether two devices are sharing the same com port:

And make sure that each time you insert a device, there must be a new COM port instance displayed in the “Device manager” menu.

## Reset Registry

If for any reason that you cannot have the device been detected correctly in your PC, one of the easiest way is remove all the setting in the registry, and then your PC will re-verify driver again for any following plugged-in device(s). Be sure to have the driver available as you install the driver for the device in the beginning.

1). From “Start” -> “Run”, enter “regedit”, and you will have Windows Registry Editor program popped up in the screen.

2). Select “HKEY_LOCAL_MACHINE” -> “SYSTEM” -> “CurrentControlSet” -> “Enum” -> “USB”, find the register tree for MT3351 device: “Vid_0e8d&Pid_0004”.

3). Press mouse over the key value, and right click to have option menu displayed, select “Usage Permission”.

4). Change permission to “full control” so that we can remove it later.

5). Press mouse over the key value again, and right click to have option menu displayed, select “Delete”.

6). Confirm the removal of key values.

7). Check the result. No key set of Vid_0e8d&Pid_0004 available any more.

8). Now insert the device, and you will be asked to provide the driver.

## USB Power

From experiment, while trying to download multiple devices at the same time, it’s very easy to get failed if the devices cannot get enough power. Directly connect all devices to PC or through a USB hub without extra power supply will cause the devices fail to download and run. We strongly suggest you use USB hub with extra power supply to connect MT3351 devices.

(MediaTek suggests using USB 2.0 HUB with power supply.)